GEC IT3 QUESTION PAPER

- Explain how the multifunction principle enhances product design

The Multifunction TRIZ Principle is one of the inventive principles in the Theory of Inventive Problem Solving (TRIZ), which focuses on creating innovative solutions to engineering problems. This principle involves designing components or systems to perform multiple functions, reducing the number of parts, simplifying the design, or increasing overall efficiency.

Here’s how it enhances product design:

1. Reduction of Complexity and Cost

By combining functions into a single component, the design becomes simpler, reducing the number of parts. This often leads to lower manufacturing and maintenance costs.

Example:

In water filtration systems, a combined filter that removes particulates and neutralizes odors eliminates the need for separate units, reducing space and cost.

2. Improved Functionality and Performance

Components designed with multiple purposes can improve overall system performance.

Example:

A heat exchanger in an HVAC system that also serves as a structural support enhances the system's functionality without adding extra components.

3. Space Optimization

Multifunctional components allow designers to achieve compact and efficient layouts, crucial for products with size constraints.

Example:

Smartphones integrate cameras, flashlights, and communication tools in a compact device, optimizing user experience and portability.

4. Enhanced Sustainability

Fewer materials are used in manufacturing multifunctional components, reducing environmental impact.

Example:

In an electric vehicle, regenerative braking components also recharge the battery, minimizing energy waste and material usage.

5. Increased Innovation Potential

Applying this principle encourages creative thinking and leads to novel solutions by reimagining the roles of existing components.

Example:

A water bottle lid that doubles as a cup adds functionality without increasing the number of components.

Practical Steps in Applying the Multifunction Principle:

Identify overlapping needs: Analyze where existing parts or systems can perform additional roles.

Integrate functions intelligently: Ensure that combining functions doesn’t compromise performance or reliability.

Leverage new materials or technologies: Use advanced materials or smart technologies to enable multifunctionality.

In essence, the Multifunction TRIZ Principle enhances product design by enabling innovation, simplifying systems, and creating cost-effective and compact solutions tailored to user needs.

- Explain how preliminary actions can prevent problems in engineering process

The Preliminary Actions TRIZ Principle focuses on taking proactive measures to address potential issues before they arise in an engineering process. It emphasizes preparing for future challenges by implementing changes, pre-emptive designs, or strategies that mitigate risks and improve efficiency.

Here’s how the principle can prevent problems in engineering:

1. Anticipating Failures and Weak Points

By identifying potential failure points in advance, engineers can design processes or systems that minimize these risks.

Example: In a water filtration system, pre-installing a sediment trap prevents clogging of finer filters downstream, reducing maintenance needs and prolonging the system's lifespan.

2. Reducing the Impact of Environmental or Operational Variability

Implementing solutions that account for external factors ensures consistent performance even under changing conditions.

Example: Coating metal components with anti-corrosion materials during manufacturing ensures long-term durability in harsh environments.

3. Streamlining Processes for Future Efficiency

Performing time-consuming or critical actions earlier in the process prevents bottlenecks later on.

Example: In automotive assembly, pre-machining engine parts with precision reduces the likelihood of misalignment issues during final assembly.

4. Enhancing Safety by Pre-empting Hazards

Proactively addressing safety concerns can prevent accidents and downtime.

Example: Installing pressure relief valves in a hydraulic system before operational testing prevents dangerous overpressure scenarios.

5. Minimizing Waste and Costs

Taking preliminary actions often reduces resource consumption and prevents costly rework or waste.

Example: Pretesting electronic components for defects ensures only functional parts are used in final assembly, reducing waste and improving reliability.

Key Strategies for Implementing Preliminary Actions

- Risk Assessment:

Analyse potential failures using tools like FMEA (Failure Mode and Effects Analysis) to guide pre-emptive measures.

Example: Predicting material fatigue in high-stress areas helps engineers reinforce designs before production.

- Scenario Simulation:

Use simulations to predict how systems respond to various stressors and address vulnerabilities proactively.

Example: Wind tunnel testing for aircraft models before full-scale manufacturing ensures designs can withstand aerodynamic loads.

- Design for Maintenance:

Embed features that make future maintenance easier and less disruptive.

Example: Inserting inspection ports in machinery allows for quick and easy access without disassembling the entire system.

- Buffering Against Future Expansion:

Design systems that can handle future upgrades or expansions without requiring complete overhauls.

Example: Building a power plant with extra capacity to accommodate future load increases.

Conclusion

By implementing the Preliminary Actions TRIZ Principle, engineers can avoid foreseeable problems, improve system reliability, enhance safety, and reduce costs. This approach fosters a proactive mindset, ensuring that designs are robust and prepared to handle future challenges.

- Describe a scenario where an initial problem led to an unexpected beneficial outcome

A scenario where an initial problem led to an unexpected beneficial outcome aligns with the TRIZ Principle of "Turning Harm into Benefit" (Blessing in disguise). This principle encourages finding ways to use a harmful effect or problem as an opportunity to create positive results.

Scenario: Car Engine Overheating Issue

Initial Problem:

An automotive manufacturer discovered that one of their car models frequently overheated during long drives, especially in hot climates. The overheating was due to inefficient heat dissipation from the engine block.

Unexpected Outcome:

During testing, engineers noticed that the excess heat could be channelled through a modified system to warm other parts of the vehicle. This led to the development of a waste heat recovery system that utilized the engine's heat to improve vehicle efficiency.

Beneficial Outcome:

- Cabin Heating Improvement: The recovered heat was directed to enhance the vehicle's cabin heating system, reducing the need for energy from the battery or fuel.

- Increased Fuel Efficiency: Some of the heat was used to preheat the engine oil, improving fuel efficiency by reducing the energy needed during cold starts.

- New Product Feature: The car was marketed with a unique selling point: "Smart Thermal Management," which became a competitive advantage in colder regions.

- Describe how separation can resolve conflicts between competing design requirements

The TRIZ Principle of Separation addresses conflicts in design requirements by resolving contradictions without compromise. This principle suggests that competing requirements, which may seem mutually exclusive, can coexist when separated by different dimensions, conditions, or perspectives.

Types of Separation:

Separation in Space: Allocate conflicting functions to different parts of the system or locations.

Separation in Time: Perform competing functions at different times.

Separation by Condition: Enable functions under different conditions (e.g., temperature, pressure).

Separation by Perspective: Change how the problem is viewed to remove the contradiction.

Example: Smartphone Battery Life vs. Processing Power

Conflict:

Smartphones face a classic design conflict between battery life and processing power. High-performance processors require more energy to function, but the more power used, the faster the battery drains. Users demand both long battery life and high processing power, which traditionally conflict.

- Separation in Space:

Instead of trying to balance both high processing power and battery life within the same components, engineers separate the functions. This was done by creating dual processor systems: one low-power processor that handles background tasks and a high-performance processor that activates only when demanding tasks (like gaming or video processing) are required.

Example: Many modern smartphones use energy-efficient cores for routine tasks and high-performance cores for intensive applications, often referred to as big. LITTLE architecture (a combination of powerful cores and energy-efficient cores).

- Separation in Time:

Another approach is to optimize power use by controlling when the high-performance processor is activated. The processor can be designed to consume more energy only during periods of high demand, such as when running intensive apps or during gaming, and to hibernate or slow down during idle or low-demand times.

Example: Dynamic Frequency Scaling allows processors to adjust their clock speed depending on the current load. Under light use, the clock speed is reduced, conserving energy.

- Separation by Condition:

Components are designed to adapt to different operating conditions, where high processing power is activated only when necessary, and lower power is used for basic tasks. This helps conserve battery life when the device is not under heavy load.

Example: The processor in a smartphone might utilize advanced power management technologies, where it can change its performance level based on battery charge and current usage conditions (e.g., dimming the screen or turning off unnecessary background apps when the battery is low).

- Separation by Perspective:

Engineers shifted the perspective of the problem by focusing on overall power efficiency rather than just maximizing either performance or battery life. By using a combination of better power management software, adaptive processors, and energy-efficient components (like OLED screens or low-power Bluetooth), they achieve a better balance between both requirements.

Example: Using an AI-powered system that adjusts power settings based on user behaviour and usage patterns, smartphones can optimize when to conserve power and when to deliver high performance.

Resulting Benefits:

- Improved Battery Life: Smartphones can now deliver high battery performance for extended periods without sacrificing performance for demanding tasks.

- Enhanced User Experience: Users get the best of both worlds powerful performance for gaming or apps while maintaining efficient battery use for everyday tasks.

- Longer Device Longevity: Devices last longer because the system intelligently uses energy, and battery cycles are optimized.

- Provide an example where symmetry change was successfully applied to solve a design problem

Example: Printer Mechanism Design

Initial Problem:

A manufacturer of laser printers faced a challenge with the paper feeding mechanism. The printer's design was based on a symmetrical mechanism that used two rollers to feed the paper through the system. However, the symmetrical roller system often led to paper jams because the paper wasn't aligned correctly when passed between the rollers. This resulted in frequent paper misfeeds, causing delays and damage to the printer.

Application of TRIZ Principle of Symmetry Change:

The Symmetry Change TRIZ principle suggests changing the symmetry of a system to better accommodate the forces acting on it. In this case, engineers noticed that the paper’s path through the printer was too rigid due to the symmetrical roller system. To resolve this, they applied an asymmetrical design to better guide the paper through the system.

Thermal transfer printer mechanisms can be considered asymmetrical due to the design and operation of their components, particularly the printhead and the movement of the media. Unlike symmetrical designs where parts are uniform and evenly distributed, asymmetrical mechanisms often have uneven distributions of weight or force, affecting the printing process and quality. This asymmetry can also influence alignment, heat distribution, and overall print results.

- Asymmetrical Roller System:

Instead of using two identical rollers to feed the paper, the engineers reversed the symmetry by using one larger, powered roller and a smaller, passive roller. This allowed the larger roller to push the paper forward with greater precision, while the smaller roller guided the paper’s path more smoothly through the printer.

- Curved Path for Paper:

The engineers also introduced an asymmetrical, curved paper path that better aligned the paper with the rollers. By slightly changing the path geometry, the paper no longer had to fight against the mechanical forces of the rollers, reducing misfeeds.

- Adjustment of Roller Pressure:

The asymmetry also allowed for differing roller pressures, where the larger roller exerted more pressure on the paper while the smaller roller applied less. This minimized the chances of the paper getting jammed between the rollers.

Beneficial Outcome:

- Reduced Paper Jams: The change from a symmetrical roller system to an asymmetrical one significantly reduced the occurrence of paper jams. The new system better aligned the paper and reduced friction, ensuring smoother paper feeding.

- Improved Printing Speed: With fewer jams and misfeeds, the printer’s overall efficiency improved, leading to faster printing times.

- Enhanced Durability: The system also reduced wear on the components, since the asymmetrical design distributed pressure more evenly and reduced strain on the rollers.

- Provide an example where inflation/ deflation mechanisms improved product performance or adaptability

Example: Inflatable Lifeboat

Initial Problem:

A company designing a lifeboat for maritime safety wanted to create a product that was compact and easy to store on ships but could inflate rapidly when needed for emergencies. The goal was to ensure the boat was lightweight and space-efficient while still being large and sturdy enough to carry multiple passengers in rough sea conditions.

Application of TRIZ Principle of Inflation/Deflation:

The Inflation/Deflation TRIZ principle was applied to the lifeboat design by incorporating an inflatable structure that could be deflated for storage and inflated quickly when needed. The principle involved using air pressure to change the size and strength of the lifeboat, adapting to both storage constraints and emergency needs.

- Deflation for Compact Storage:

The lifeboat was designed to be deflated for storage, which allowed it to be stored in a compact form within a small, easily accessible compartment on the ship. The deflated boat could be rolled up and packed into a small, lightweight container, minimizing storage space.

- Inflation for Full-Size Functionality:

In the event of an emergency, the lifeboat could be rapidly inflated using compressed air or a CO2 cartridge. Once inflated, the boat expanded into its full size, offering sufficient capacity to carry the required number of passengers and withstand rough sea conditions. The boat’s shape and structure were designed to maximize buoyancy and stability once inflated, ensuring safety even in harsh conditions.

- Automatic Inflation Mechanism:

The boat’s inflation system could be triggered automatically by an emergency release mechanism or manually activated in case of a slower evacuation process. The boat inflated within seconds, ensuring minimal delay in emergency situations.

Beneficial Outcome:

- Compact and Efficient Storage: The lifeboat could be stored in small compartments, reducing space usage on ships and making it easier to deploy multiple boats if necessary.

- Rapid Deployment: The lifeboat could be deployed almost instantly in emergencies, which is crucial for saving lives in situations where time is of the essence.

- Sturdy and Safe: Once inflated, the lifeboat was strong enough to handle the weight of several people and provide a stable platform in rough seas.

- Provide an example where color was strategically used to enhance product appeal or performance

The Color TRIZ principle involves using color strategically to enhance product appeal, function, or user experience. In this case, the manufacturer applied color in several ways to improve both the aesthetic appeal and functionality of the smartphone.

Example: Smartphone Design with Color-Coding for Functionality

A smartphone manufacturer wanted to make their phone both visually appealing and easy to use.

  - Color-Coding Functional Elements:

- To help users easily identify essential buttons and features, the smartphone utilized color differentiation for different parts of the device:

- The power button was highlighted in a vibrant color (e.g., red) to make it easy to find quickly.

- The volume buttons were differentiated using a contrasting color (e.g., blue) to distinguish them from other buttons.

- Ports such as USB and charging ports were color-coded with specific hues (e.g., green for charging and yellow for audio) to reduce user errors in connecting accessories.

  - Enhanced Visual Appeal:

- The smartphone was released in a variety of bold, eye-catching colors, such as metallic blue, matte black, and bright red, appealing to different user preferences and helping the product stand out in the market.

- The use of gradient color effects on the phone’s back panel (e.g., a fading effect from blue to purple) made the device visually striking, attracting attention from consumers who value aesthetics.

  - Ergonomics and Color for Accessibility:

- For accessibility, the phone used a high-contrast color scheme on the screen, helping users with visual impairments. For instance, important icons and text were rendered in high-contrast colors (e.g., white text on a dark background) to improve readability.

- Illustrate examples showcasing successful compositions leading towards desired outcomes across industries

The Composite TRIZ principle suggests combining two or more materials with complementary properties to achieve the desired result.

Engineers in the aerospace industry successfully applied this principle by using composite materials—combinations of materials like carbon fibre reinforced polymer (CFRP)—to achieve both strength and lightness.

- Composite Materials (CFRP):

Carbon fiber reinforced polymer (CFRP) composites were used to create lighter, stronger parts for the aircraft. Carbon fibers provide high strength while maintaining low weight, while the polymer matrix holds the fibers together and gives the material flexibility and durability.

- Innovative Material Layering:

Engineers designed multilayered composite structures, combining various materials to take advantage of their specific properties. For instance, the outer layers of the fuselage may be made of a tough, durable material to withstand harsh weather, while the inner layers are made from lighter, insulating materials for energy efficiency.

- Tailored for Different Conditions:

The composite materials were designed to handle specific loads and stresses. For example, different composite materials were used for various parts of the aircraft (such as wings, fuselage, and engine components) to ensure optimal performance based on the specific mechanical demands of each part.

Example 1: Aerospace – Composite Materials in Aircraft Construction

The aerospace industry faced a conflict between the need for lightweight materials for fuel efficiency and strength for safety and durability in aircraft construction. Traditional materials like steel or Aluminium were heavy, and lighter alternatives often didn't offer the required strength.

Example 2: Automotive – Composite Brake Pads

In the automotive industry, manufacturers faced the challenge of designing brake pads that offered high performance and durability. Traditional metal-based brake pads wore out quickly, while ceramic pads were expensive and didn't provide as much stopping power.

Beneficial Outcome:

- Lighter Aircraft: The composite materials reduced the weight of the aircraft significantly, leading to improved fuel efficiency and reduced operational costs.

- Increased Durability and Safety: The materials provided superior strength and resistance to fatigue, improving the aircraft's lifespan and ensuring passenger safety.

- Cost Savings: The use of composite materials helped reduce fuel consumption, leading to long-term cost savings for airlines.

- Analyse how phase transformations affect material properties during manufacturing processes

The Phase Transformation TRIZ principle involves altering the state of a material (e.g., solid, liquid, gas, plasma) or its internal structure (e.g., crystalline phases) to achieve desired properties during manufacturing processes.

Key Examples of Phase Transformations in Manufacturing:

1. Steel Hardening through Heat Treatment

- In steel manufacturing, phase transformations occur during heat treatment processes such as quenching, tempering, or annealing.

- When steel is heated to high temperatures and then rapidly cooled (quenching), it undergoes a transformation from austenite (a high-temperature phase) to martensite (a hard, brittle phase).

- By tempering (controlled reheating), the martensite transforms partially, balancing hardness and toughness.

Material Property Changes:

- Before Phase Transformation: Steel is softer and more ductile in its annealed state.

- After Phase Transformation: The steel becomes much harder and wear-resistant due to the formation of martensite, though tempering reduces brittleness for practical use.

Applications: Used in manufacturing cutting tools, gears, and automotive components that require high strength and wear resistance.

2. Aluminium Alloy Strengthening through Precipitation Hardening

- Aluminium alloys are strengthened by a process called precipitation hardening or age hardening.

- The alloy is first heated to a high temperature to dissolve the alloying elements into a single-phase solution (solid solution).

- It is then rapidly cooled (quenched), trapping the alloying elements in a supersaturated state.

- During aging (at room or elevated temperatures), precipitates (secondary phases) form within the metal matrix, enhancing strength.

Material Property Changes:

- Before Phase Transformation: Aluminium is softer and lacks strength for structural applications.

- After Phase Transformation: The fine precipitates act as obstacles to dislocation movement, dramatically improving the material’s strength and fatigue resistance.

Applications: Widely used in aerospace and automotive industries for lightweight yet strong components.

3. Glass Manufacturing – Controlled Crystallization in Glass Ceramics

- Glass ceramics are created through a controlled crystallization process, where amorphous glass is partially transformed into a crystalline structure by heat treatment.

- This process combines the transparency of glass with the strength and thermal resistance of ceramics.

Material Property Changes:

- Before Phase Transformation: Glass is brittle and prone to cracking under thermal stress.

- After Phase Transformation: The partially crystalline structure provides enhanced thermal shock resistance, strength, and durability.

Applications: Used in cookware, dental materials, and heat-resistant telescope mirrors.

4. Semiconductor Manufacturing – Silicon Crystal Growth

- Semiconductor-grade silicon undergoes a phase transformation from molten silicon to a single-crystal structure using the Czochralski process.

- As the molten silicon cools, it transitions into a highly ordered crystalline phase critical for electronic properties.

Material Property Changes:

- Before Phase Transformation: Molten silicon lacks the structural order required for electronic devices.

- After Phase Transformation: The single-crystal silicon phase provides the uniform electronic properties necessary for efficient semiconductor performance.

Applications: Used in the production of microchips and photovoltaic cells.

General Impacts of Phase Transformations on Material Properties:

- Strength and Hardness: Transformations often produce harder and stronger phases (e.g., martensite in steel, precipitates in alloys).

- Ductility and Toughness: Proper control of transformations can balance hardness with ductility (e.g., tempered steel).

- Thermal Resistance: Crystallization processes in materials like glass ceramics improve resistance to thermal stress.

- Electrical Properties: Phase transformations in semiconductors determine electronic performance, critical for modern devices.

- Analyze challenges faced when implementing recycling processes within manufacturing systems

### Challenges in Implementing Recycling Processes in Manufacturing Systems

Recycling processes are essential for sustainability but come with numerous challenges in the context of manufacturing systems. These challenges span technical, economic, logistical, and regulatory dimensions.

Material Complexity: Products often use composites or alloys, making separation and recovery difficult, reducing material purity and efficiency.

High Energy Use: Recycling processes like smelting or chemical treatment consume significant energy, offsetting environmental benefits.

Economic Viability: High costs of recycling systems and fluctuating market prices for recycled materials deter adoption.

Sorting Logistics: Contamination and inefficiencies in collection and sorting increase operational costs and waste.

Regulatory Hurdles: Compliance with varying waste management laws increases complexity, especially for hazardous materials.

Technological Gaps: Recycling technologies for complex materials (e.g., multi-layer plastics) are underdeveloped or costly.

Downcycling: Recycled materials often degrade in quality, limiting their usability in high-performance applications.

Infrastructure Limitations: Space and integration issues make scaling recycling within manufacturing challenging.

- Evaluate the trade-off involved in applying segmentation to complex systems

The Segmentation TRIZ principle involves dividing a system or process into smaller, independent parts to improve flexibility, performance, or adaptability. While segmentation offers many benefits, it also introduces trade-offs that must be carefully evaluated in complex systems.

Segmentation can enhance flexibility, reliability, and maintainability in complex systems but at the cost of increased integration complexity, efficiency losses, and higher upfront expenses. Evaluating these trade-offs is critical to achieving an optimal balance between performance and practicality.

Advantages of Segmentation:

- Improved Flexibility

- Enhanced Reliability

- Customizability

- Ease of Maintenance

Examples of Segmentation Trade-Offs:

Modular Electric Vehicles (EVs):

Benefit: Battery packs in EVs are segmented for easy replacement or upgrades.

Trade-Off: Complex thermal management and electrical connections reduce energy efficiency and increase manufacturing costs.

Building Construction:

Benefit: Prefabricated modular units speed up construction and allow customization.

Trade-Off: Structural joints may be weaker than monolithic designs, requiring additional reinforcement.

- Assess trade-offs involved when choosing dimensions based on performance requirements versus cost constraints

### Trade-Offs in Choosing Dimensions: Performance vs. Cost

When designing systems or products, selecting dimensions that balance performance requirements and cost constraints is a key challenge. Larger or more robust dimensions may enhance functionality but increase costs, while minimizing dimensions to reduce costs can compromise performance.

Performance Focus: Larger dimensions or higher-quality materials increase durability and load-bearing capacity.

Cost Trade-Off: Expensive materials and added weight lead to higher manufacturing and transportation costs.

- Shivam wants to file a patent for organo electronic contact lenses explain the three methods studied by you to help him evaluate whether he should file his patent

1. Patentability Assessment

This method ensures Shivam’s invention meets the legal requirements for a patent: novelty, non-obviousness, and usefulness.

- Prior Art Search: Investigate existing patents and publications to ensure his invention is unique. Focus on innovations in organo-electronics and smart contact lenses.

- Novelty Check: Confirm the lens includes a new feature, such as enhanced biocompatibility, superior functionality, or innovative integration of organo-electronics.

- Non-Obviousness: Verify that the invention is not an obvious improvement over existing technology.

- Utility Validation: Ensure the contact lenses have practical applications (e.g., medical diagnostics, augmented reality).

If Shivam’s invention meets these above criteria, it’s worth considering a patent.

2. Market Analysis

Understanding the market potential of the invention is critical to determining its value and impact.

Industry Trends: Analyze demand for smart wearable devices, particularly in healthcare, vision correction, and augmented reality.

Competitor Analysis: Identify competitors and their patented technologies to evaluate market positioning and differentiation.

Target Applications: Assess applications like real-time health monitoring, glucose sensing for diabetics, or AR-enhanced lenses for gaming and navigation.

If the technology serves a growing or underserved market, filing a patent can provide competitive advantages.

3. Cost-Benefit Analysis

Patenting is expensive and time-consuming; this method helps weigh the investment against potential gains.

Cost Estimation: Account for filing fees, attorney costs, and maintenance fees, which can range from $5,000 to $20,000 or more.

Revenue Potential: Estimate earnings from licensing, product sales, or partnerships. Consider exclusivity benefits of a patent.

Risk Assessment: Evaluate risks like patent infringement litigation, rapid obsolescence, or challenges in manufacturing and scaling.

If the financial and strategic benefits outweigh costs, filing a patent is justified.

Shivam should file the patent if:

- Patentability: The lenses meet novelty, non-obviousness, and utility requirements.

- Market Demand: There’s strong potential for adoption and commercialization.

- Cost Justification: The patent investment aligns with projected financial and strategic gains.

- Explain the three criteria that shivam’s invention disclosure application should satisfy to be patentable

1. Novelty

- The invention must be new and not publicly disclosed in any form before the filing date.

- Shivam’s contact lenses must not duplicate any prior inventions, patents, or published works in the field of organo-electronics or smart lenses.

- Even slight differences from existing technologies can establish novelty if they result in distinct features (e.g., a unique integration of organo-electronics for medical diagnostics or enhanced user comfort).

- Conduct a prior art search to check for similar technologies or ideas already patented or published.

2. Non-Obviousness

- The invention must involve an inventive step that is not obvious to someone skilled in the relevant field.

- Shivam’s invention should demonstrate a significant improvement or innovative concept that goes beyond a routine enhancement of existing technologies.

- For instance, integrating organo-electronics into contact lenses in a way that improves energy efficiency, sensor functionality, or comfort must solve a unique problem or overcome a known limitation.

- Assess whether an expert in the field of wearable electronics would consider the invention as an unexpected or innovative development.

3. Utility

  - The invention must have practical applicability and solve a real-world problem.

  - Shivam’s contact lenses should have a clear and specific purpose, such as health monitoring (e.g., glucose levels), augmented reality applications, or vision correction with enhanced features.

  - The utility must be demonstrable and not merely theoretical.

  - Prove the functionality with prototypes, research data, or feasibility studies showing that the lenses meet their intended purpose.